GO WORKSHEET FOR ASSESSMENT OF OFFICE SPACE REQUIREMENTS (Agency)

(Country Office)

Part 1: Space Allocation per Staff Member / Grade / Agency

| Post Title | Post Grade | (A) Sq. M per | (B) Sq. Ft per | (C) No. of persons | Total Net Space by level (A x C = Sq.Mt) (B x C = Sq.Ft) |
| --- | --- | --- | --- | --- | --- |
| Regional Director | D2 | 24.00 | 258 |  |  |
| UN Resident Representative | P5-D1 | 17.84 | 192 |  |  |
| All Intern. Staff | P1-P5 | 8.90 | 96 |  |  |
| National Staff, JPO, UNV | All | 8.90 | 96 |  |  |
| GS Staff (excluding drivers) | All | 5.94 | 64 |  |  |
| Drivers – common area |  | 2.33 | 25 |  |  |
| Subtotal of Space for personnel |  |  |  |  |  |
| Multiply by Circulation Factor (20%) |  |  |  |  |  |
| Total Space for Personnel (P) |  |  |  |  |  |

Notes:

- Space allocation as per JIU

- WC / Toilet facilities are included in circulation

- Space for drivers in not per person but rather common use room

4.    WC Facilities: For estimate purpose the area of WC is considered to be included in the circulation percentage. For design purpose the following standards may be used:

|  | Per 25 staff | Minimum per office |
| --- | --- | --- |
| Men | 1 WC + 1 urinal + 2 WBs | 2WCs + 2 urinals + 2 WBs |
| Ladies | 2 WCs + 2 WBs | 2 WCs + 2WBs |

Part 2:Space Allocation for Common Facilities / Agency

| Facilities | Space calculated per | Sq. M | Sq. Ft | No. of staff / unit | Total Net Space |
| --- | --- | --- | --- | --- | --- |
| Conference Room | Staff member | 1.40 | 15 |  |  |
| Library | Staff member | 0.55 | 6 |  |  |
| Common File Room | Staff member | 0.65 | 7 |  |  |
| Reception | Office | 9.0-18.0 | 97-194 | N/A |  |
| Communication Room | Office | 9.0-13.5 | 97-145 | N/A |  |
| Audio Visual Storage Room | Office | 9.0-18.0 | 97-194 | N/A |  |
| Supply Room | Staff member | 0.25 | 3 |  |  |
| Central Reproduction Room | Staff member | 0.50 | 5 |  |  |
| Copy Room | Machine | 3.00 | 32 |  |  |
| Computer/LAN Room | Office | 9.0-18.0 | 97-194 | N/A |  |
| Janitor Room | Office | 4.5-9.0 | 48-97 | N/A |  |
| Eating Facilities | Staff member | 0.50 | 5 |  |  |
| Kitchen | Staff member | Up to 0.20 | 2 |  |  |
| Subtotal of Space for Common Facilities |  |  |  |  |  |
| Multiply by Circulation Factor (15%) |  |  |  |  |  |
| Total Space for Common Facilities (CF) |  |  |  |  |  |

GRAND TOTAL SPACE REQUIREMENT (Part 1 + Part 2)

Total Space Personnel (P) + Total Space for Common Facilities (CF)